COVENANT BUILDERS

WE DELIVER

876 47th Avenue, Vero Beach, FL 32966  ·  (772) 473-7115  ·  info@covenantbuilders.org
Florida Certified Building Contractor CBC1253676

Job submittal SOP — roofing

Internal use. Every step refers to the Covenant Builders CRM.

Job approval checklist

Before submitting a job for approval, make sure every required document is uploaded to the job in the CRM. When it is complete, send it for review from the job’s Documents section.

Required documents

Financial worksheet — all fields completed accurately.

Photos — roof and property only. Do not put checks in the photos folder.

Roofing agreement (final contract) — signed by both the project manager and the homeowner. Type the information; avoid handwriting wherever possible.

Signed NOC and permit documents — a Notice of Commencement is always required. Permit documents are not always needed. Request whatever is missing from the office.

Eagle View report — order the premium report; it is what material orders are built from. Order reports for the primary structure and any detached structures.

Job submittal form — every field completed.

Insurance paperwork — insurance claims only. Does not apply to retail jobs.

Check submittal — upload both a scanned copy of the check and a check submittal form. Attach the scanned check to the check submittal record, and put both in the job’s Documents section.

Job submittal form — field guide

Complete this from the template in the CRM. Do not handwrite responses.

Steep slope, low slope, pitch

Steep slope squares — any roof section with a pitch of 3/12 or greater.

Low slope squares — under 3/12; treated as flat and needs rolled roofing, not shingles.

Pitch — the primary slope, found on page 1 of the Eagle View.

Roof covering, brand, profile, colour

Covering — shingles, metal, tile, etc.

Brand — shingles: IKO, GAF, CertainTeed, Owens Corning. Tile: Eagle.

Profile — the product line, e.g. Timberline (GAF), Dynasty (IKO), Duration (Owens Corning).

Colour — as chosen by the homeowner.

Underlayment, valleys, drip edge

Underlayment — standard for shingles is Resisto peel-and-stick; name anything different.

Valleys — state whether valley metal or Resisto.

Drip edge — colour. Default white; say so if brown, black or other.

Flat areas

This is the low slope squares. Give material and colour, or “match shingle colour”. Example: CertainTeed Flintlastic Weathered Wood.

Attic ventilation

Ridge vent — total feet required.

Off-ridge vent (ORV) — number of pieces and colour, e.g. “4, Brown”.

Pipe jacks and goosenecks

Leave blank or enter 0 if none.

Skylights

We primarily use Kennedy and Velux. Give brand, glass type (tempered or high impact), curb size (2” or 4”), mounting (self-flashing or curb-mounted), dimensions and quantity.

Example: 3 Kennedy skylights, self-flashing, high impact glass, 2” curb.

Electrical intake, satellite dish, chimney, chimney cricket

Leave blank or enter “no/0” if none. If a satellite dish is being discarded, write “trash” here or in Notes.

Known leaks, problems, bad wood

Be specific so the crew can find and fix it. If extra plywood is needed, give a quantity.

Existing gutters

Blank means the existing gutters stay. If replacing, say whether it is a full wrap or partial (name the areas), and give the colour.

Paver driveway

Yes or no.

Dumpster location

Where the homeowner wants it placed.

Gate code

Required for crew access, deliveries and inspections. Without it an inspection can fail.

Solar

Whether panels are present, and whether they are to be removed and reinstalled or discarded.

Flashing

Blank means none will be ordered. “Replace any bad flashing” orders one piece; give a quantity if more is needed.

Total structures

Matters for permitting. State whether there is one main residence or multiple detached structures, make sure all appear on the Eagle View, and flag if a separate material order is needed.

Notes and instructions

Anything that did not fit above, or anything worth emphasising.

Example: “We are using GAF UHDZ, not just HDZ” or “Order 5 total plywood.”

Check handling

Create a check submittal

Open the job in the CRM

Go to Documents

Add a document into the Job Paperwork folder

Choose Check submittal

Save

For 10% draws, send a note to the office rather than submitting directly.

Upload a check from the field

Open the job on your phone

Go to Documents

Add a document and scan it

Name it clearly, then save

Attach a check to an existing check submittal

Open the job, then Documents

Open the check submittal

Edit, add document, choose the scanned check from the job’s documents

Save